9-60V              12-72V           40A           800W

Input Voltage              Output Voltage            Current              Power

Key Features

•  Wide range input voltage

•  Design meeting RoHS / CE

•  Maximum output power: 150W

•  Maximum efficiency: 95%

•  Support CAN or 485 bus communication function(Note2)

•  Support input overvoltage protection

•  Output short circuit limiting protection,

overtemperature protection,input voltage undervoltage protection

•  Waterproof level IP65

•  Voltage can be raised or lowered

•  Filled with full glue,Natural air cooling

•  Dual external fuse design for both input and output

• 2 Years warranty

Usage Environment

- •  Industrial Automation Machineryxing

- •  In the field of new energy

- •  Industrial control system

- •  Energy storage charging system

- •  RV and ship control system

- •  Electronic instruments, equipment, and

- devices

Description

EC-XXH151 is an efficient DC charger with a wide input voltage range that can provide stable output voltage. This charger has the advantages of small and beautiful size, good heat dissipation performance, and the open protocol supports intelligent control.

。

Model Description

This charger module has 485 or Can communication

for users to choose from. Users can interact with the charging module through a protocol, obtain basic information about the charging module, and set the module's UVP、OVP、OTP、Control module power on/off, etc.

Model Naming Method

- E：”Enerchg”,company abbreviation

- C：charger,product type

- 3.  10：10A,Maximum continuous output current

- H:boost - buck

- 151：output max power，15 * = 150w

- M: shell material

Electrical specifications

| Model No. |  |  | EC-10H151M | EC-05H151M |
| --- | --- | --- | --- | --- |
| Output | Switching frequency |  | 100Khz | 100Khz |
|  | Default Voltage |  | 14.6V | 29.2V |
|  | Default Current |  | 10A±3%↓ | 5A±3%↓ |
|  | Default Power |  | 150W↓ |  |
|  | Voltage Charging Range |  | 10-15V | 16-30V |
|  | Regulator accuracy(Typ.) |  | ±3% | ±3% |
|  | Voltage regulation(Typ.) |  | ±2% | ±2% |
|  | Load Regulation(Typ.) |  | ±1% | ±1% |
|  | Output ripple and noise |  | 640mVp-p | 500mVp-p |
|  | Output voltage rise time |  | 171ms | 171ms |
|  | Boot delay time |  | 321ms | 321ms |
|  | Output external capacitance |  | 4000μF | 3000μF |
|  | Positive electrode cable |  | Suggestion: 20AWG Note5 | Suggestion: 22AWG Note5 |
|  | Negative electrode cable |  | Suggestion: 20AWG Note5 | Suggestion: 22AWG Note5 |
|  | Output fuse |  | 15A | 7.5A |
| Input | Efficiency Note3 | 12V | 95% | 95% |
|  |  | 24V | 96% | 96% |
|  | Voltage Range |  | 10-40Vdc |  |
|  | Max. input voltage |  | 42V |  |
|  | No-load power range |  | 1.0~1.5W |  |
|  | Max. input current |  | 15A |  |
|  | Input capacitance |  | 1320μF |  |
|  | Positive electrode cable |  | Suggestion: 18AWG Note5 |  |
|  | Negative electrode cable |  | Suggestion: 18AWG Note5 |  |
|  | Input fuse |  | 20A |  |
| Protect | Input UVP |  | UVP:<9Vdc Recover:>9.5Vdc (Def.) |  |
|  | Input OVP |  | OVP:>43Vdc Recover:<42Vdc (Def.) |  |
|  | Overload protection |  | > 150W (Def.) Hiccup Mode |  |
|  | Output Short Circuit |  | Hiccup Mode |  |
|  | Over Temperature |  | 105℃Protect/ 95℃Recover (Def.) |  |
| Function | RS485 or CAN Communication Note4 |  | Set battery charging voltage, current, power, temperature, UVP and OVP, LED full charge conversion point, report charging information and faults. For details, please refer to RS485/CAN communication protocol |  |
|  | LED light display |  | RED: battery is charging; GREEN： fully charged |  |
| Environment | operating temperature |  | -40~55℃ |  |
|  | operating humidity |  | 5~90% |  |
|  | storage temperature |  | -55~100℃ |  |
|  | Atmospheric pressure |  | 62~106Kpa |  |
|  | Altitude |  | 2000m |  |
|  | Cooling way |  | Natural cooling |  |
| Safety And EMC Features | Anti-electric Strength |  | Input to Output | ≥1500Vdc |
|  |  |  | Input to Shell | ≥1500Vdc |
|  |  |  | Output to Shell | ≥500Vdc |
|  | Insulation resistance |  | Input to Output | ≥10MΩ |
|  |  |  | Input to Shell |  |
|  |  |  | Output to Shell |  |
| Other | MTBF |  | ≥100,000H |  |
|  | Weight |  | ≤1kg |  |
|  | Package |  | White Box |  |
|  | Dimension |  | 140*65*25mm(L*W*H) |  |
|  | Shell material |  | aluminum alloy and insulating plastic |  |
|  | Casting material |  | UL 94V-0 |  |
| Note | - Unless otherwise specified, all specification parameters are measured under the conditions of an input of 24V direct current (dc), rated load, and an ambient temperature of 25℃.For details, please refer to the "EC-XXH151 test report". - It can be selected when purchasing. It doesn't support both CAN and 485. - For details, please refer to the "efficiency curve". - For details, please refer to the "EC-XXH151 protocol Specification". - The recommended value of the wire is the minimum requirement. If the wire length exceeds 50 cm, please use a thicker wire. - ※Product Disclaimer: |  |  |  |

Characteristic Curves （TA = 25℃ Vin = 24V）

| Figure 1, Efficiency curve |  |
| --- | --- |
| Vout=14.6V,Iout=10A | Vout=29.2V,Iout=5A |
| Figure 2, Power dissipation |  |
| Vout=14.6V,Iout=10A |  | Vout=29.2V,Iout=5A |  |
| Figure 3, Input V-I |  |  |  |
| Vout=14.6V,Iout=10A |  | Vout=29.2V,Iout=5A |  |

Typical Waveforms （TA = 25℃ Vin = 24V）

| Figure 4, 50% - 75% load dynamic |  |
| --- | --- |
| Vout=14.6V,load 50%-75%-50% | Vout=29.2V,load 50%-75%-50% |
| Figure 5, Output ripple & noise |  |
| Vout=14.6V,Iout=10A | Vout=29.2V,Iout=5A |
| Figure 6, Output voltage established |  |
| Vout=14.6V,Iout=10A | Vout=29.2V,Iout=5A |
| Figure 7, Boot delay time |  |
| Vout=14.6V,Iout=10A | Vout=29.2V,Iout=5A |
| Figure 9, Short circuit & Out voltage |  |

Feature Description

| Input Under - Voltage Protection | Input Over - Voltage Protection |
| --- | --- |
| The converter will shut down after the input voltage drops below the under voltage protection threshold for shutdown. The converter will start to work again after the input voltage reaches the input under voltage protection threshold for startup. For the Hysteresis, see the Protection characteristics. |  |
| Overload protection | Output Short Circuit |
|  | A temperature sensor on the converter senses the average temperature of the module. It protects the converter from being damaged at high temperatures. When the temperature exceeds the over temperature protection threshold, the output will shut down. It will allow the converter to turn on again when the temperature of the sensed location falls by the value of Over temperature Protection Hysteresis |
| Over Temperature |  |
| A temperature sensor on the converter senses the average temperature of the module. It protects the converter from being damaged at high temperatures. When the temperature exceeds the over temperature protection threshold, the output will shut down. It will allow the converter to turn on again when the temperature of the sensed location falls by the value of Over temperature Protection Hysteresis |  |

Thermal Consideration

Mechanical Diagram